Whole House Water Purification Survey

When you contact us please let us know:  (complete then contact us)

What type of water you wish to treat?

Undersink 		  Whole House 		  Well 		  City/county piped in water

What do you not like about your water?

What water quality do you wish after treatment?

Is there an existing automatic sprinkler system or usage other than home (whole house only)?

How many gallons per day are used, and how many people are using the water?

What is your complete address?

How many gallons per minute does your well pump yield (whole house well treatment only)?

GPM Recovery Test (whole house well treatment only)

Tools needed: Accurate pressure gauge, 5-gal bucket,

Watch.

Make sure water system is at full pressure (psi).

Make sure no water is being used during the test.

Turn off the well pump.

Drain water into the bucket until 0 psi.

How many gallons did you drain?

Close the drain valve.

Turn on pump and time in seconds until the pump turns off.

How many seconds did the pump run?

Formula to figure GPM Recovery rate:

Gallons divided by 	 seconds X 60 = 	 GPM

- Repeat to ensure accuracy

Eco-$mart, Inc. (888) 329-2705 ph / (941) 377-9460 fx / info@eco-smart.com

# Instructions for your

Water Test

Find a clean 16 oz (or larger) plastic bottle.

Run your raw untreated water for 5 minutes.

Fill with raw untreated water.

Send us the water sample, this form with the section above completed, & $25.  Mail to:

Eco-$mart Whole House Ozone Water Test

410 Municipal Drive

Leander, TX 78641

(512) 259-7575

You will receive a $25 credit when the system is purchased, so the test becomes FREE.

- Fill a 12oz glass of sulfur water.

- Smell it immediately to verify there is sulfur present

- Hold it away from you nose and move 10’ away from the sink.

- Wait 1 minute with the glass held away from you so you cannot smell the sulfur.

- Smell again after 1 min.

- If you cannot smell it you have < 1.5 ppm (generally).

- If you can still smell sulfur: Move 10’ away from that spot and hold the glass away from you for another minute.

- Smell again.

- If gone you have < 3 ppm. If still present a formal test from one of your LaMotte Sulfur Test Kits will be necessary.

This test kit accurately measures up to 20 ppm sulfur and has 50+ tests capability.

The kit costs $75 including shipping.

### Rule of thumb Sulfur Test